Rental Web Site notes;

California concentrates not by name.

A Lodi winery in California that needed grape juice filtration capacity trialed a centrifuge along with a 800mm 30 cu.ft. Pac-Press filter press for one season during their grape crushing period.  The filter press used filter aid to precoat the filter. A small amount of filter aid was also used in the feed of the juice as it was being filtered as a bodyfeed. The customer was able to filter 11,500 gallons of juice which contained about 2% spinnable solids before they needed to discharge the solids. The cycle time took just over 3 hours to process and roughly 40 minutes to clean the filter press which included washing the cloths after each cycle. The  client decided the filter press was a better fit over the decanter centrifuge as the solids retention by the filter press was far superior. The purchase of a decanter would have required a second filtration process to capture the entrained solids, costing more overall. The client purchased a filter press 3 times the capacity of the rental which easily scaled up.

City of Redlands- not by name.

A City Municipality was using the traditional method of settling ponds to capture suspended solids as the first step of treatment from water to used for drinking water in the community. Mountain run off water would enter one side of the pond, by the time it reached the opposite side of the pond the suspended solids settled in the pond. The pond was used continuously until the level of the solids in the pond became so high that the solids started to carry over into the water exiting the pond. At that point the municipality would switch to a different pond that had enough capacity for the solids. The pond with the solids would be manually pumped as dry as possible which left a liquid slurry. A bulldozer was then used to push the solids out of the lagoon onto a flat area nearby that allowed the water to continue to drain from the solids.  The solids dried in the sun, it took months to clean out the pond and get it back to a capacity where it would be able to process water again. The client field tested a filter press to ensure that the solids could be trapped in a filter press and could create a dry cake. A 90 day testing period included the help of the local chemical supplier that provided a coagulant and a polymer to ensure the solids would be able to be removed and would not blind off the filter press. As a result of the testing the client was so pleased they purchased a full scale filter press for the municipality. The full scale unit was 10 times larger than the rental pilot unit. This scaling up of the full size unit from the rental test was reasonable based on the results achieved.

GAF/BASF

A decommissioned Bayer chemical manufacturing site in upstate New York had years of industrial contamination in a local lagoon that was adjacent to the Hudson River. The site had several million gallons with an accumulation of solids to be removed from the ponds to be remediated and properly disposed of.  In the first phase of the project, Pacific Press Company provided 2 each 100 cu.ft. rental filter trailer presses that were used 24/7 for almost one year as a temporary treatment. In conjunction with several “Rain for Rent” clarifiers and sand filters the filter presses handled the solids that were precipitated out of the contaminated waste streams. The filter presses were trailer mounted with Air Operated diaphragm pump skids to feed the solids into the filter press for dewatering. The customer supplied portable air compressors that provided air for operating the AOD pumps as well as the air blow down feature on the filter presses. Once the solids were dewatered in the filter press the solids were discharged onto a drag chain conveyor where they were discharged out the back of the trailer into a refuse bin for further consolidation and disposal.

Tunnel Boring

A Pacific Press Company filter press was rented for 24 months in the temporary processing of Bentonite-laden spoil slurry during the Tunnel Boring Machinery process in the Seattle Washington Tunnel. The tunnel boring machine know as “Big Bertha” an EPB machine was used to tunnel the new SR-99 highway in downtown Seattle. The wash down water from the process treated with a Flocculant then pumped into clarifiers where the settled solids were pumped into a filter press. The filter press resided in a temporary building that was located directly over a roll off bin. The ability of the Pacific Press filter press to dewater this solution was the final step in the water treatment system.